# Guide to Connect UniFi Access Points to a Cloud Controller

## Prerequisites

- UniFi Cloud Controller (e.g., hosted on Ubiquiti's cloud, AWS, or a remote VM)

- Access point powered on and connected to a network with internet access

- Access to the Cloud Controller's public IP or hostname

- Default or factory-reset UniFi AP

## Step 1: Power and Connect the AP

- Connect the AP to a PoE switch or injector.

- Ensure the network has DHCP and internet access.

## Step 2: Identify the AP

- Option 1: SSH into the AP

- - Use a network scanner (e.g., Advanced IP Scanner or nmap) to find the AP’s IP.

- - SSH into the AP: ssh ubnt@<AP-IP> (default password: ubnt)

- Option 2: Use UniFi Mobile App (Optional)

- - The app can detect new APs on the same local network and adopt them if cloud access is configured.

## Step 3: Point the AP to the Cloud Controller

- Once in the AP via SSH, run the following command:

  - set-inform http://<Cloud-Controller-IP-or-FQDN>:8080/inform

  - example:  set-inform http://xxxxxxxx-xxxx-xxxx-xxxx-xxxxxxxxxxxx.unifi-hosting.ui.com:8080/inform

## Step 4: Adopt the AP in the Cloud Controller

- Log into your Cloud Controller.

- Open Google Chrome - https://xxxxxxxx-xxxx-xxxx-xxxx-xxxxxxxxxxxx.unifi-hosting.ui.com/network/default/dashboard

- Go to Devices.

- You should see the new AP listed as 'Pending Adoption'.

- Click Adopt.

## Troubleshooting Tips

- Make sure ports 8080 (inform), 443 (web), and 3478 (STUN) are open inbound on the controller.

- Confirm APs can reach the controller’s hostname or IP.

- Factory-reset AP with: syswrapper.sh restore-default

- Press and hold reset pin on AP until flashing white and blue

- Then Unplug network cable to the AP

- Delete from Cloud control then reconnect with Putty and then Adapt